AtmoLink

ESP32 Weather Station

Technical Documentation & Developer Guide

DHT22  •  SSD1306 OLED  •  OpenWeatherMap  •  ESP-IDF

# 1. Project Overview

AtmoLink is a self-contained ESP32 firmware application that combines local ambient sensing, an I²C OLED display, a live web dashboard, and cloud weather reference data into a single cohesive system. It is written entirely in C using the ESP-IDF framework and FreeRTOS.

Core capabilities:

- Reads temperature and humidity from a DHT22 sensor every 3 seconds.

- Renders live values on a 128×64 SSD1306 OLED over I²C.

- Hosts a dark-themed, mobile-responsive HTTP dashboard on port 80.

- Polls the OpenWeatherMap (OWM) current-weather API every 10 minutes and displays the delta between local and cloud readings.

- Uses FreeRTOS tasks and a mutex for safe concurrent data access.

# 2. Hardware Requirements

## 2.1 Bill of Materials

| Component | Specification | GPIO / Interface |
| --- | --- | --- |
| ESP32 DevKit | Any ESP32 module (WROOM-32 recommended) | — |
| DHT22 / AM2302 | Temperature + Humidity sensor | GPIO 4 (DATA) |
| SSD1306 OLED | 128×64 pixel, I²C, 3.3 V | SDA: GPIO 21 / SCL: GPIO 22 |
| Pull-up resistor | 4.7 kΩ (DHT22 data line) | 3.3 V to GPIO 4 |
| Power supply | USB or 3.3 V regulated | VIN / 3.3 V pin |

## 2.2 Wiring Diagram

Connect the components as follows:

DHT22  DATA  →  GPIO 4  (+ 4.7kΩ pull-up to 3.3V)

DHT22  VCC   →  3.3V

DHT22  GND   →  GND

OLED   SDA   →  GPIO 21

OLED   SCL   →  GPIO 22

OLED   VCC   →  3.3V

OLED   GND   →  GND

# 3. Software Architecture

## 3.1 Task Overview

AtmoLink runs four concurrent execution contexts inside FreeRTOS:

| Task / Context | Stack | Priority | Period | Responsibility |
| --- | --- | --- | --- | --- |
| task_dht | 4 096 B | 5 (highest) | 3 s | Read DHT22; update g_sensor under mutex |
| task_oled | 8 192 B | 4 | 2 s | Read shared data; render OLED frame |
| task_owm | 12 288 B | 3 | 10 min | HTTP GET to OWM; parse JSON; update g_owm |
| HTTP server | 16 384 B | N/A | On request | Serve HTML dashboard at GET / |

## 3.2 Shared State & Synchronization

Two global structs hold the application state:

sensor_data_t g_sensor  —  { temperature, humidity, valid }

owm_data_t    g_owm     —  { owm_temp, owm_humidity, owm_desc, owm_valid }

A single FreeRTOS mutex (g_mutex) guards both structs. Every read or write takes the mutex, performs the operation, then immediately releases it.

## 3.3 Module Breakdown

SSD1306 OLED Driver

A lightweight, self-contained driver is embedded directly in main.c. It maintains a 1 024-byte frame buffer (128 columns × 8 pages) and uses a built-in 5×7 pixel ASCII font. The oled_flush() function transmits the entire buffer to the display in 16-byte I²C chunks.

WiFi Station

Standard ESP-IDF WiFi STA initialization with event-group synchronization. Up to 5 reconnect attempts are made before setting WIFI_FAIL_BIT. IP assignment triggers WIFI_CONNECTED_BIT, which unblocks app_main.

OWM HTTP Client

Uses esp_http_client to issue a single GET request to the OWM current-weather endpoint with metric units. The response body is accumulated in a 2 048-byte static buffer. Temperature, humidity, and weather description are extracted with lightweight string-search helpers (parse_float / parse_str) — no JSON library required.

Web Server

An esp_httpd instance listens on port 80. The single handler (root_handler) assembles the HTML response in three chunked sends to avoid large stack allocations. The page carries a 5-second meta-refresh for live updates without JavaScript.

# 4. Configuration Reference

| Macro | Default | Description |
| --- | --- | --- |
| WIFI_SSID | "ssid" | 2.4 GHz network name |
| WIFI_PASSWORD | "psw" | WPA/WPA2 passphrase |
| OWM_API_KEY | "yourapikey" | OpenWeatherMap API key (free tier) |
| DHT_GPIO | GPIO_NUM_4 | GPIO pin connected to DHT22 DATA |
| CITY_LAT | "16.7107" | Latitude for OWM query (Eluru default) |
| CITY_LON | "81.0952" | Longitude for OWM query (Eluru default) |
| I2C_MASTER_SDA | GPIO_NUM_21 | OLED SDA pin |
| I2C_MASTER_SCL | GPIO_NUM_22 | OLED SCL pin |
| I2C_MASTER_FREQ | 400 000 Hz | I²C bus frequency (Fast Mode) |
| SSD1306_ADDR | 0x3C | I²C address (try 0x3D if blank) |
| WIFI_MAX_RETRY | 5 | Max WiFi reconnect attempts |

# 5. Sample Outputs

## 5.1 Serial Monitor (ESP-IDF Log)

Expected output after power-on with valid credentials:

I (3201) atmolink: IP: 192.168.1.42

I (5212) atmolink: GPIO4 idle: 1 (must be 1)

I (8412) atmolink: DHT22: 31.4 C  72.3%

I (9053) atmolink: OWM: 30.1 C  68.0%  overcast clouds

I (9054) atmolink: Web server up on port 80

W (14500) atmolink: DHT22 err: ESP_ERR_TIMEOUT   ← transient; recovers next cycle

## 5.2 OLED Display Layout

The display refreshes every 2 seconds across 8 pages (rows):

Page 0:     AtmoLink

Page 1:  -------------

Page 2:  T: 31.4 C

Page 3:  H: 72.3%

Page 4:  OWM ref:

Page 5:  T: 30.1 C

Page 6:  dT: +1.3 C

Page 7:  (unused)

## 5.3 Web Dashboard

Navigate to http://<device-ip>/ in any browser. The page auto-refreshes every 5 seconds and is mobile responsive. It shows:

- DHT22 live card grid: Temperature and Humidity in large blue numerals.

- OWM reference card grid: cloud temperature and humidity in green.

- Delta row: ΔT and ΔH difference between DHT22 and OWM.

- Weather description string from OWM (e.g., "overcast clouds").

## 5.4 OWM API Response (parsed fields)

AtmoLink extracts three fields from the JSON response:

{

"main": { "temp": 30.1, "humidity": 68 },

"weather": [{ "description": "overcast clouds" }]

}

# 6. Build & Flash Instructions

## 6.1 Prerequisites

- ESP-IDF v5.x installed and sourced (. $IDF_PATH/export.sh)

- dht22.h / dht22.c placed in components/dht22/

- Valid OWM API key (free tier at openweathermap.org)

## 6.2 Commands

git clone https://github.com/your-username/atmolink.git

cd atmolink

idf.py set-target esp32

idf.py build

idf.py -p /dev/ttyUSB0 flash monitor

# 7. Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| DHT22 err: ESP_ERR_TIMEOUT repeatedly | Missing or wrong pull-up resistor | Add 4.7 kΩ from DATA to 3.3 V; check wiring |
| OLED stays blank after boot | Wrong I²C address | Change SSD1306_ADDR to 0x3D |
| OWM shows Fetching… indefinitely | Invalid API key or WiFi not connected | Verify key; check serial for IP assignment |
| Web page not reachable | Firewall or wrong IP | Check serial monitor for assigned IP |
| Stack overflow / reboot loop | Task stack too small | Increase xTaskCreate stack sizes in app_main |
| WiFi FAIL shown on OLED | Wrong SSID/password or 5 GHz network | Use 2.4 GHz band; verify credentials |

# 8. License

This project is released under the MIT License. You are free to use, copy, modify, and distribute the code for any purpose, provided the copyright notice is retained.

Acknowledgements:

- OpenWeatherMap — free weather data API

- Espressif ESP-IDF — ESP32 development framework

- SSD1306 font bitmaps adapted from the Adafruit GFX Library